N.NITHISH

192512300

Ex.No:1: Simulation of FDMA in MATLAB

Aim: To simulate a Frequency Division Multiple Access (FDMA) system using MATLAB and analyze its communication performance in terms of channel bandwidth, data rate, and signal-to-noise ratio (SNR).

Objective 1:

To simulate an FDMA system by allocating individual frequency channels to multiple users and determine the channel bandwidth assigned to each user.

Output Parameter: Channel Bandwidth (kHz) Objective 2:

To calculate the data rate achieved by each user in the FDMA system based on the allocated channel bandwidth.

Output Parameter: Data Rate (kbps) Objective 3:

To evaluate the communication quality of the FDMA system by calculating the Signal-to-Noise Ratio (SNR) for different users.

Output Parameter: Signal-to-Noise Ratio (SNR) (dB) Procedure

- Open MATLAB and create a new script file.

- Define the total available bandwidth, number of FDMA users, carrier frequencies, signal power, and noise power.

- Allocate a unique frequency channel to each user and calculate the channel bandwidth.

- Compute the data rate for each user based on the allocated bandwidth and modulation scheme.

- Calculate the Signal-to-Noise Ratio (SNR) using the signal and noise power values.

- Plot the transmitted signals of all FDMA users using different carrier frequencies.

- Display the calculated channel bandwidth, data rate, and SNR in the MATLAB Command Window.

- Analyze the results to verify independent channel allocation and evaluate the overall performance of the FDMA system.

Objective 1 Program

clc; clear; close all; fs=1000; t=0:1/fs:1; TBW=600; Users=3;

CBW=TBW/Users; fc=[100 300 500];

m=sin(2*pi*10*t); s1=m.*cos(2*pi*fc(1)*t); s2=m.*cos(2*pi*fc(2)*t); s3=m.*cos(2*pi*fc(3)*t); subplot(2,1,1) plot(t,s1,'r',t,s2,'g',t,s3,'b'),grid on

title('FDMA Channel Allocation'),xlabel('Time (s)'),ylabel('Amplitude') subplot(2,1,2)

bar(1:Users,CBW*ones(1,Users)),grid on

xlabel('Users'),ylabel('Channel Bandwidth (kHz)'),title('Channel Bandwidth per User')

output

Objective 2 Program:

clc; clear; close all;

BW = 200; % Channel Bandwidth (kHz) Users = 3;

M = 2; % BPSK Modulation

DataRate = BW*log2(M); % Data Rate (kbps) Rate = DataRate*ones(1,Users);

disp(' User Data Rate (kbps)') disp([(1:Users)' Rate'])

figure bar(1:Users,Rate) xlabel('Users')

ylabel('Data Rate (kbps)')

title('FDMA Data Rate for Each User') grid on

output:

Objective 3:

Program:

clc; clear; close all;

BW = 200; % Channel Bandwidth (kHz) Users = 3;

M = 2; % BPSK Modulation

DataRate = BW*log2(M); % Data Rate (kbps) Rate = DataRate*ones(1,Users);

disp(' User Data Rate (kbps)') disp([(1:Users)' Rate'])

figure bar(1:Users,Rate) xlabel('Users')

ylabel('Data Rate (kbps)')

title('FDMA Data Rate for Each User') grid on

Result:

- The FDMA system was successfully simulated by allocating separate frequency channels to three users, and the channel bandwidth assigned to each user was determined.

- The data rate achieved by each user was successfully calculated based on the allocated

channel bandwidth, demonstrating equal transmission capacity for all users.

- The Signal-to-Noise Ratio (SNR) for different users was successfully evaluated, showing that higher SNR values provide better communication quality and reliable data transmission.